15.01.2010

HAREKET Proje Taşımacılığı

Specification for chassis

| Variant Description | TGS 33.540 6X6 BBS |
| --- | --- |
| Basic vehicle number: | L56SLJ61-00000 |
| Type of vehicle: | Semitrailer tractor, Conventional |
| Cab: | LX Cab with high roof |
| Wheelbase: | 3600 mm |
| Overhang: | 0700 mm |
| Steering: | L |
| Front-axle load: | 8000 Kg |
| Rear axle load: | 26000 Kg |
| Perm. gross weight | 33000 Kg |

Tyres

| Front axle | 2 * 81#R0060387 GY 315/80R22,5 OMNIT MSS TL156/150 K Strg-S + G |  |
| --- | --- | --- |
| Rear axle | 4 * 81#R0060329 GY 315/80R22,5 OMNIT MSD 156/150 K Drive-S + G |  |
| 2nd rear axle | 4 * 81#R0060329 GY 315/80R22,5 OMNIT MSD 156/150 K Drive-S + G |  |
| Spare wheel | 1 * 81#R0060387 GY 315/80R22,5 OMNIT MSS TL156/150 K Strg-S + G |  |

Colours

| chassis | 9011 graphite black RAL 9011 matt | W |  |
| --- | --- | --- | --- |
| wheels | 9006 White aluminium acrylic RAL 9006 | N |  |
| cab | 9010 Pure white RAL 9010 | N |  |

Customer specification: equipment for vehicle TGS 33.540 6X6 BBS / L56SLJ61-00000

Engine Cooling System Clutch

| 018JPM | MAN 6-cylinder 4-valve in-line engine D2676LF06, with Common Rail injection, turbocharging and intercooling, EDC engine regulation, rating 540 hp/397 kW at 1900 rpm, torque 2500 Nm at 1050 - 1400 rpm, low-pollutant exhaust gases to EURO5 standard. SCR catalyst and AdBlue injection. With on-board diagnosis system (OBD2) and NOX control. |  |
| --- | --- | --- |
| 124ARM | additional fuel filter heated with water trap, make SEPAR |  |
| 203ESM | Steuermodul f. ext. Datenaustausch (KSM)m. Flotten-Management-Systemschnittstelle(FMS)Aufbaufunktionalität |  |
| 210AAM | flame starting system |  |
| 211AFM | Twin plate clutch MFZ 2/400X Only suitable for GVW more than 90t and up to 160 t or 180 t. Because of the higher mass moment of inertia of this clutch a shorter service life of the synchronising equipment in the gearbox must be expected. |  |
| 280EUM | Stone protection grille in front of radiator |  |

Intake / Exhaust System

| 201ARM | The raised air intake is on the lh side of the cab rear wall and is integrated in the rear wall contour. Thus no loss of body length. The air filter is in the centre of the vehicle under the cab above the top edge of the frame. The intake duct contains a pre-separator for the intake air. This equipment is available for EU countries too from 1st quarter 2004. |  |
| --- | --- | --- |
| 201EMM | air filter element with safety element for heavy-duty operation |  |

Gearbox

| 022WVM | Gearbox 16S252 OD, 16 forward speeds, 2 reverse (please note fast reverse gear is blocked). Synchromesh gearbox, ratios 13.80 - 0.84 with Intarder (extra weight approx. 74 kg), for GTW up to 160 t (see heavy duty transport requirements information). Single-H shift lever to prevent incorrect shifting. Moving off possible only in low range. |  |
| --- | --- | --- |

Transfer case

| 146KLM | MAN-Verteilergetriebe G 253 mit Geländegang, Übersetzung Straßengang 1:0,981, Geländegang 1:1,582. Vorteile: Größerer Achsabstand für geringere Beugewinkel der Kardanwellen. Längere Ölwechsintervalle wegen eingebautem Ölfilter. Mit Neutralstellung für Bet |  |
| --- | --- | --- |

Power Take-Offs

| 120AXM | double-flange coupling, elastic |  |
| --- | --- | --- |
| 120KHM | PTO on flywheel side with 6-hole flange (100D) f=1.233, 870 Nm continuous load - 950 Nm brief load |  |

Front Axle / Front Springs / Front Axle Load

| 026ECS | Parabolic front springs, 8.0 t, with progressive characteristics and shock absorbers, 2-leaf version (status 11/05). (Note: picture shows 3-leaf design). |  |
| --- | --- | --- |
| 534FAW | Permissible FA load 8000 kg. Note tyre load rating. |  |

Rear Axle / Rear Springs / Rear Axle Load

| 028FPM | rear springs parabolic 16 t |  |
| --- | --- | --- |
| 034MHS | MAN hub reduction rear tandem axle AP HPD-1382/HP-1352 Gross train weights: for I <= 4.0: 70 t (for heavy-duty transport for up to 10 % 160 t, otherwise up to 120 t). For i > 4.0: 82 t (for heavy-duty transport up to 250 t). See heavy-duty transport fitting requirement information. For actual axle loads see load variants. |  |
| 128EBM | Pressure sensor for steel sprung vehicles with a gross combined weight rating of over 120 tons |  |
| 535KSW | Permissible RA load 2 x 13000 kg. Note tyre load rating. |  |

Transmission ratios

| 035KNW | planetary axle ratio i = 6.00 |  |
| --- | --- | --- |

Fuel Tanks

| 023JUM | Fuel tank, aluminium 600l right and 35l AdBlue |  |
| --- | --- | --- |
| 123CCM | strainer for fuel tank |  |
| 303AAM | tank cap lockable - 1 cap w/o vent |  |

Frame

| 493YTS | JOST fifth-wheel base plate 40 mm (fifth-wheel loads up to 36t) |  |
| --- | --- | --- |

Trailer/Fifth-wheel Coupling

| 491CFM | Jost JSK 38 C-1 2" fifth-wheel coupling. Height 190 mm, max. fifth-wheel load 28000 kg, D value 170 kN, weight approx. 190 kg, connecting hole pattern to DIN 74081/74084 and ISO 3842. |  |
| --- | --- | --- |
| 493MPW | kingpin to rear axle centre 1075 mm |  |

Brake System

| 034ECS | brake shoe width for rear axle 220 mm |  |
| --- | --- | --- |
| 044CVM | Brake cylinders for 44 t gtw and over. Spring-loaded brake cylinder 30" instead of 24" |  |
| 370CRM | air dryer, heated |  |

Cab exterior

| 050NMS | LX cab, 2240 mm wide, 2280 mm long, with high roof. Many storage facilities as standard. Very large interior for maximum comfort even with two drivers. |  |
| --- | --- | --- |
| 388AKM | The sun visors are fastened with 4 high-strength plastic brackets. At the side they are also supported by the mounts for the marker lights. The visor brackets are prepared for additional attachments. |  |

Cab interior

| 153EYM | EBERSPÄCHER AIRTRONIC D4S is a diesel/air heater with a maximum heat output of 3.5 kW. Temperature can be set at a clock with thermostat function. Heat output is controlled in 4 stages from 1000 to 3500 W. The heater is under the co-driver's footwell. The |  |
| --- | --- | --- |
| 153KCM | Air-conditioner, CFC-free refrigerant 134 A. Automatic temperature control. With front crankshaft drive, 2-groove belt pulley and refrigerant compressor. |  |
| 376FCM | 2 Schlafliegen mit Stauraum (Alu-Rahmen) für den Einsatz von 2 Fahrern im internationalen Fernverkehr. Beide Schlafliegen sind klappbar und mit einer Sicherungsschiene (unten) und mit einem Sicherungsnetz (oben) versehen. Unter der unteren Schlafliege be |  |
| 376HNW | mattress for upper bunk |  |
| 378ALW | 2 pillows |  |
| 390ATM | The storage box is stowed under the bunk and can be pulled out. It has two compartments inside: up to three 2 l bottles will fit in the first compartment, while the compartment behind can be used for other utensils.. |  |

Instruments

| 042FYM | EC tachograph electronic, 2 drivers, with add. recorder (no EU-approval since 2006) |  |
| --- | --- | --- |
| 325EZW | language 1 'Turkish' for display in instrument panel |  |

Electrical System

| 327AFM | battery main switch, mechanical |  |
| --- | --- | --- |
| 329CTM | 2 batteries, 12V 175Ah |  |
| 331HCM | alternator 28V 110A 3080W |  |

For Special Vehicles

| 280ACW | wiring for tank truck to ADR |  |
| --- | --- | --- |

Other

| 194AWW | operator's manual in TURKISH |  |
| --- | --- | --- |
| 405CPM | 20 m tyre inflator hose with pressure gauge |  |
| 407AHS | jack 12 t |  |

Auxiliary equipment ex works

| ZW011M | OD transmission is only poss. with a gross vehicle weight greater than 6 |  |
| --- | --- | --- |
| ZW012M | OD transmission is only poss. with a gross vehicle weight greater than 6 |  |
| ZW013M | OD transmission is only poss. with a gross vehicle weight greater than 6 |  |
| ZW014M | OD transmission is only poss. with a gross vehicle weight greater than 6 |  |
| ZW015M | OD transmission is only poss. with a gross vehicle weight greater than 6 |  |
| ZW016M | OD transmission is only poss. with a gross vehicle weight greater than 6 |  |
| ZW017M | OD transmission is only poss. with a gross vehicle weight greater than 6 |  |

İlktuğ Çağatay Kepek

MAN Kamyon ve Otobüs Tic. A.Ş.

Vehicle data Power train  - TGS 33.XXX 6X6 BBS   L56SLJ61-HAREKET

| Truck train: | Semitrailer train |
| --- | --- |
| GTW [kg] | 44000 |
| Design speed [km/h]/engine speed at design speed [rpm] | 89/no value |
| Residual climbing ability in the highest gear at design speed [%] | no value |

Engine

| Sales code | Designation | Performance | Start of green speed range | End of green speed range | Max. speed |
| --- | --- | --- | --- | --- | --- |
| 018JP | engine D2676LF06 - 540 hp / 397 kw EURO5 SCR - 2500 Nm C-R OBD2 | 540 | 1305 | 1440 | 1900 |

Gearbox

| Sales code | Designation |  |  |  |  |  | Transmission intervals |  |  |  | max. GCW |  |  |  | Forward gears |  |  |  | Reverse gears |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 022WV | gearbox ZF 16 S 252 OD with ZF intarder |  |  |  |  |  | 13,8-0,84 |  |  |  | 160000 |  |  |  | 16 |  |  |  | 2 |  |  |
| Forward gears |  | 1 | 2 | 3 | 4 | 5 |  | 6 | 7 | 8 |  | 9 | 10 | 11 |  | 12 | 13 | 14 |  | 15 | 16 |
| Transmission |  | 13,80 | 11,54 | 9,49 | 7,93 | 6,53 |  | 5,46 | 4,57 | 3,82 |  | 3,02 | 2,53 | 2,08 |  | 1,74 | 1,43 | 1,20 |  | 1,00 | 0,84 |

Rear axle

| Sales code | Designation | Design | Load capacity | Transmission | max. GCW |
| --- | --- | --- | --- | --- | --- |
| 034MH | hub-reduction rear axle HPD-1382/ HP-1352 | AP | 26000 | 6.00 | 70000 |

Tyres (rear axle)

| Tread pattern | Sales group | Diameter | Circumference | Single load | Double load | SI |
| --- | --- | --- | --- | --- | --- | --- |
| GY 315/80R22,5 OMNIT MSD 156/150 K Drive-S + G | 81#R0060329 | 1093 | 3333 | 4000 | 3350 | K |

Traction

| Skid point [%] | 40 | Condition of the ground | Road |
| --- | --- | --- | --- |
| Moving-off behaviour in the lowest gear [%] | 29 | at eng. speed [rpm] | 900 |

Speed (On-road gears): at max. eng. speed

Climbing ability (On-road gears): Road

Chassis - TGS 33.XXX 6X6 BBS   L56SLJ61-HAREKET

| Cab: | LX Cab with high roof |
| --- | --- |
| Wheelbase/overhang | 3600/700 |

Semitrailer:

| Width: | 2240 |
| --- | --- |
| Height: | 2525 |
| Max. overall height: | 4000 |
| Distance to cab: | 240 |
| Weight of fifth-wheel coupling: | 135 |
| Semitrailer weight: | 0 |
| Max. payload (calculated): | 22510 |

Chassis:

| Load capacity: | 22645 |
| --- | --- |
| Perm. weight: | 33000 Kg (8,0/13,0/13,0) |

Tyres:

| Diameter: | 1093 |
| --- | --- |
| tFA: 315/80R22.5 | 0 Kg Change tyres |
| tRA: 315/80R22.5 | 0 Kg Change tyres |

Pickup plate:

| Fifth-wheel height unladen/laden | 1296/1226 |
| --- | --- |
| Fifth-wheel coupling height: | 185 |
| Fifth-wheel lead: | 1025 |

Table of axle loads:

| Weight distribution | Centre of gravity to tFA | at front (kg) | % | at rear (kg) | % | total (kg) | % |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Chassis incl. change of tyres |  | 5635 | 17,0 | 4720 | 14,3 | 10355 | 31,3 |
| Fifth-wheel coupling |  | 32 |  | 103 |  | 135 |  |
| Load capacity / payload | 3975 | 1712 | 5,1 | 20933 | 63,4 | 22645 | 68,6 |
| Body empty 4840 | 3975 | 0 | 0 | 0 | 0 | 0 | 0 |
|  |  | ====== |  | ====== |  | ====== |  |
| Overall load |  | 7379 | 22,3 | 25756 | 78,0 | 33135 | 100,4 |
| Max.axle loads |  | 8000 |  | 26000 |  | 33000 |  |
| Load reserve (+)/overload (-) |  | 621 |  | 244 |  | -135 |  |
|  |  | ---------- |  | ---------- |  | ---------- |  |
| Chassis dead weight (standard) |  |  |  |  |  | 10355 |  |

Payload diagram - TGS 33.XXX 6X6 BBS   L56SLJ61-HAREKET

Heights sheet - TGS 33.XXX 6X6 BBS   L56SLJ61-HAREKET

Length diagram - TGS 33.XXX 6X6 BBS   L56SLJ61-HAREKET